# Design Document – Firmware Installer

## 1. System Architecture Overview

TekMedika Firmware Updater is a comprehensive tool designed to simplify firmware installation for Arduino Mega and NodeMCU boards. The system is divided into three main components:

- Installer: A Python-based Desktop GUI application that provides an intuitive interface for users to install firmware on master and slave boards as well as update or install UID into the system.

- Backend Server: A Node.js/Express-based server that records firmware installation details and provides APIs for interaction.

- Database: A MySQL database that stores firmware installation records, including system UID, firmware version, and timestamps.

### High-Level Architecture

## 2. Components

### 2.1 Installer

- Technology: Python (Tkinter for GUI)

- Purpose: Provides a user-friendly interface for firmware installation, version selection, and board management.

- Key Features:

  - Detects connected boards (Master/Slave) via USB.

  - Downloads firmware files dynamically from a GitHub repository.

  - Installs firmware on the boards using arduino-cli and esptool.

  - Displays progress and logs for each step.

  - Registers firmware installation details with the backend server.

### 2.2 Backend Server

- Technology: Node.js with Express.js

- Purpose: Handles API requests from the installer and interacts with the database.

- Key Features:

  - Records firmware installation details (system UID, firmware version, timestamp).

  - Provides endpoints for retrieving installation history.

  - Secures API interactions using a hashed secret key.

### 2.3 Database

- Technology: MySQL

- Purpose: Stores firmware installation records.

- Schema:

  - Table: firmware_installations

| Field | Type | Description |
| --- | --- | --- |
| id | INT (PK, AUTO_INCREMENT) | Unique record ID |
| system_uid | VARCHAR | Unique identifier for the system |
| firmware_version | VARCHAR | Installed firmware version |
| installation_timestamp | TIMESTAMP | Timestamp of the installation |
| verification_status | BOOLEAN | Whether the installation was verified |

## 3. Firmware Installation Flow

### 3.1 Home Page – Update Check

- When the application launches, users are directed to the Home Page.

- The app automatically checks for the latest version using the firmware/appReleases API.

- If a newer version is available:

  - A notification is shown to the user with a prompt to update.

- After the update check, users are directed to the Firmware Selection Page.

### 3.2 Admin Login

- On the Firmware Selection Page, an "Login" button is available.

- Clicking the button opens a login prompt with the following credentials:

  - Email: admin

  - Password: admin

- On successful login, the user enters Admin Mode with access to additional features:

  - Ability to select and install any firmware version.

  - Access to Flash Erase functionality for both boards.

  - Advanced options become visible in the UI.

### 3.3 Firmware Selection

- On the Firmware Selection Page, users select the firmware version from a dropdown menu.

  - Default User Mode: Can only install the latest firmware version.

  - Admin Mode: Can select any available firmware version.

- The selected firmware files are dynamically downloaded from the GitHub repository.

### 3.4 Master Board (Arduino Mega) Installation

- The application detects the connected Arduino Mega board using serial.tools.list_ports.

- Using arduino-cli, the following actions are performed:

  - Upload the compiled firmware to the board.

- After installation, the app verifies the operation by:

  - Extracting the System UID and Firmware Version via serial communication.

  - Displaying the verification status to the user.

### 3.5 Slave Board (NodeMCU) Installation

- The application detects the connected NodeMCU board using serial.tools.list_ports.

- Using arduino-cli, the following actions are performed:

  - Flash the selected firmware file (.bin) to the board.

- After installation, the app verifies the operation by:

  - Extracting the System UID and Firmware Version via serial communication.

  - Displaying the verification status to the user.

### 3.6 Flash Erase (Admin Only)

- Admin Mode users can access the Flash Erase feature for both boards.

- The process includes:

  - Uploading a Empty file to the Master and Slave boards to clear their flash memory.

  - Receiving confirmation of the successful erase operation from each board.

3.7 UID Installation Flow

- UID Installation Feature:

  - Accessed from the Version Selection Page.

  - Requires Admin login.

- UID Download Process:

  - System downloads UID installation firmware from GitHub.

  - Progress is displayed.

- Connection Instructions:

  - Users connect Master board (Arduino Mega) to computer.

  - Safety warnings regarding power are displayed.

- UID Installation:

  - Firmware uploads to Arduino Mega.

  - System automatically generates a unique UID during installation.

- Barcode Generation:

  - Upon success, a Code128 barcode is generated for the UID.

  - Barcode can be saved for future reference.

- API Integration:

  - If a previous UID existed, it's deprecated in the database (verification_status set to false).

  - The new UID is registered in the database.

- Completion:

  - Success screen confirms UID installation and displays new UID.

  - Users can return to the main firmware selection screen.

## 4. UID & Version Extraction Logic

- Purpose: To uniquely identify the system and verify the installed firmware version.

- Process:

  - Establish a serial connection with the board.

  - Send specific commands:

    - H: Handshake to ensure communication.

    - U: Retrieve the system UID.

    - V: Retrieve the firmware version.

  - Parse the responses and return the extracted values.

- Implementation: The logic is implemented in the get_device_info function in api.py.

## 5. API Flow and Database Interaction

### 5.1 API Endpoints

- POST /api/firmware-installation

- Purpose: Record firmware installation details.

- Request Body:

  - {
  "system_uid": "unique-system-id",
  "firmware_version": "x.x.x",
  "verification_status": true, 
}

  - Response: Success or error message.

- GET /api/firmware-installation/:system_uid

  - Purpose: Retrieve firmware installation history for a specific system UID.

  - Response: List of installation records.

- POST /api/deprecate-uid

  - Purpose: Deprecate previous UID records (sets verification_status to false).

  - Request Body:

{

"old_system_uid": "unique-system-id-to-deprecate",

"secret": "hashed-secret-key"

}

  - Response: Count of deprecated records

- GET /download-excel

  - Purpose: Downloads all firmware installation records in Excel format.

  - Response: File: firmware_installations.xlsx

  - Excel Columns:

ID

System UID

Firmware Versio

Installation Timestamp

Verification Status (shown as Verified or Not Verified)

### 5.2 Backend GUI

#### /view-installations

- Purpose: Renders an HTML table view of all firmware installation records.

- Displayed Columns:

  - ID

  - System UID

  - Firmware Version

  - Installation Timestamp

  - Verification Status

### 5.3 Database Interaction

- Insert Operation: When a firmware installation is completed, the installer sends a POST request to the backend, which inserts the details into the firmware_installations table.

- Update Operation: When a new firmware is installed on an existing system, the backend updates the existing record with the new firmware version.

- Deprecate Operation: When a new UID is installed, all previous records for the old UID are marked as deprecated by setting verification_status to false.

- Query Operation: The backend retrieves installation history for a given system UID using a SELECT query.

## 6. Project Structure

TekMedika-Firmware-Updater/

│── main.py # Main entry point of the application │── src/ │ ├── arduino/ # Arduino-specific scripts │ │ ├── arduino.py │ ├── nodemcu/ # NodeMCU-specific scripts │ │ ├── nodemcu.py │ ├── ui/ # UI components │ │ ├── home_page.py │ │ ├── login_page.py │ │ ├── version_selection_page.py │ │ ├── download_screen.py │ │ ├── run_installation.py │ │ ├── erasing_pages.py │ │ ├── utils.py │ │ ├── assets/ # UI assets │ ├── uid_installation/ # UID installation components │ │ ├── barcode_screen.py │ │ ├── completion_screen.py │ │ ├── download_screen.py │ │ ├── uid_connection_instructions.py │ │ ├── uid_installation_screen.py │ ├── backend/ # Backend API logic │ │ ├── api.py │ ├── config/ # Configuration files │ │ ├── config.py │ │ ├── color.py │ ├── utils/ # Utility functions │ │ ├── utils.py │ │ ├── erasing.py │ ├── bin/ # Firmware binary files │ │ ├── InstallSystemUid.ino.mega.hex │ │ ├── empty/ │ │ ├── empty.ino │── backend/ # Node.js backend server │ ├── app.js │ ├── package.json │ ├── views/ │ │ ├── home.ejs │ │ ├── table.ejs │── requirements.txt # Python dependencies │── setup_script.iss # Setup script for installer

## 7. UI Flow

### 7.1 Home Page

- Purpose: Welcome screen with a brief description of the tool.

- Actions: Navigate to the login page.

- Home Page when the app is Latest

- Home Page when the app is not Latest

### 7.2 Version Selection Page

- Purpose: Allow users to select the firmware version to install.

- Actions:

  - Admins can access additional erase options.

  - Users can proceed to download the selected firmware.

- Default Version Selection Page

- Version Selection Page after admin login

### 7.3 Login Page

- Purpose: Authenticate users (Admin).

- Actions: Validate credentials and navigate to the version selection page.

### 7.4 Download Screen

- Purpose: Display download progress for firmware files.

- Actions: Proceed to connection instructions after download completion.

### 7.5 Connection Instructions

- Purpose: Guide users to connect the appropriate board (Master/Slave).

- Actions: Start the installation process.

### 7.6 Installation Progress

- Purpose: Show progress of firmware installation.

- Actions: Display logs and progress bar.

### 7.7 Completion Screen

- Purpose: Confirm successful installation.

- Actions: Exit the application or restart the process.

7.8 UID Installation Screens

7.8.1 UID Selection from Version Page

•	Purpose: Provide access to the UID installation feature.

•	Actions: Admin users can select the "Install System UID" button and trigger the UID installation flow.

7.8.2 UID Download Screen

•	Purpose: Download the required UID installation firmware.

•	Actions: Display progress of the UID firmware download and automatically proceed to the connection instructions upon completion.

7.8.3 UID Connection Instructions

•	Purpose: Guide users to connect the Master board properly.

•	Actions: Provide clear instructions with safety warnings, And Allow users to proceed to installation or go back.

7.8.4 UID Installation Screen

•	Purpose: Display the UID installation process.

•	Actions: Show real-time logs of the installation steps and display progress information.

7.8.5 UID Completion Screen

•	Purpose: Confirm successful UID installation.

•	Actions: Display confirmation message with the installed UID and allow users to return to the main version selection screen.

## 8. Python Dependencies (with Versions)

altgraph==0.17.4

certifi==2025.1.31

charset-normalizer==3.4.1

future==1.0.0

idna==3.10

iso8601==2.1.0

packaging==24.2

pefile==2023.2.7

pillow==11.1.0

psutil==7.0.0

pyinstaller==6.12.0

pyinstaller-hooks-contrib==2025.2

pyserial==3.5

pywin32-ctypes==0.2.3

PyYAML==6.0.2

requests==2.32.3

setuptools==78.1.0

urllib3==2.3.0